����������������������������������������������������������������������ͻ

� IDE11 V2.30 - Integrated Development Environment for 68HC11          �

����������������������������������������������������������������������Ķ

� Copyright (C)1994-1996 by MCT Lange & Thamm Mikrocomputertechnik GbR �

� All Rights Reserved                                                  �

����������������������������������������������������������������������ͼ

I. What is IDE11?

�����������������

IDE11 is an Integrated Development Environment for Motorola's 68HC11

family of microcontrollers. IDE11 makes it easy for you to create

and debug HC11 assembly language applications. IDE11 includes:

Editor

- Multi File Editor

- Wordstar-like keyboard commands

- Cut/Copy/Paste/Clipboard

- Search/Find/Replace

- Undo

Assembler

- 2-Pass Assembler (w/o Linker)

- Include files

- Complex Expression Syntax

- Additional Assembler Directives for higher compatibility

- Cooperates with Software Simulator TESTE68 (map file creation)

- Assembly Listing

- S-Record Output

Download

- Download to internal/external RAM/EEPROM

- Remote Reset Control

- Remote Mode Control

- Load, execute (load & start), goto (restart)

- Optional support for EPROM simulator

- Capable of downloading S-Records created by 3rd-Party Tools

Debugging

- Single Step Trace in internal/external memory w/o additional

hardware (method: timer interrupt)

- Memory Browser (option: write to file)

- Disassembler Window (option: write to file)

Terminal

- Various user baud rates (10417, 9600, 7812, 4800, 2400, 1200, 600 Bd)

- Terminal Window (Option: Log File)

- Auto Activate after Download

Target

- Any HC11A/E/F

- 8 MHz xtal frequency

- Special Bootstrap Mode

- Expanded Mode (user definable RAM/EEPROM ressources)

- Manual reset / automatic (remote) reset via RS232 handshake line

Utilities

- Display / modify Config Register

- Display Memory Map (location of int/ext memory ressources,

registers etc.)

Misc

- Pull-Down Menus, Windows, Mouse Support

- Borland/Turbo Look&Feel (Turbo Vision)

- Various predefined targets, user definable target

- Presettable display timeout for message windows (to avoid clicking

"OK" again and again)

II. Hard- & Software requirements:

����������������������������������

IDE11 requires an '286 or better CPU and DOS (4.01 up) to run. There

are no special requirements for graphics cards (VGA / text mode).

A mouse or other pointing device is recommended. There are problems

using IDE11 in a multitasking environment. In particular you should

avoid running IDE11 under Windows 3.1. On the other hand, WfW 3.11

and Win 95 seem to work fine. The reason for this restriction is

IDE11's non-standard handling of the serial port due to the need of

special baud rates.

The target microcontroller must run with 8 MHz chrystal speed. Any

HC11A/E/F target can be used, as long as the serial interface (SCI)

is available for the purpose of communication with IDE11. Both

internal and external memory can be used. Connect the MODA and MODB

pins to ground, even if your application finally will run in Expanded

Multiplexed Mode.

III. Installation:

������������������

1. Create a subdirectory on your hard disk for IDE11 and change

to this directory, e.g.:

c:

cd \

md ide11

cd ide11

2. Unpack the archive file IDE11V23.ZIP into this directory using

UNZIP or PKUNZIP:

unzip a:\ide11v23

---------------------------------------------------------------

NOTE: IDE11 is packaged using Info-ZIP's compression utility.

The installation requires UnZip to read files from the archive.

Info-ZIP's software (Zip, UnZip and related utilities) is free

and can be obtained as source code or executables from various

anonymous-ftp sites, including ftp.uu.net:/pub/archiving/zip/*.

---------------------------------------------------------------

3. The unzip program will create all necessary subdirectories.

The following directory structure is recommended (though you

could use another scheme):

C:\

|____IDE11

|____BIN

|____LIB

|____SRC

4. Include C:\IDE11\BIN to the PATH statement in your AUTOEXEC.BAT:

PATH=C:\DOS;C:\BIN; ... ;C:\IDE11\BIN

5. Reboot your computer to let the above modification take effect

6. Start IDE11 by invoking

IDE11

This will launch IDE11 using COM1 for communication with a

target. If you prefer COM2, simply add a command line option:

IDE11 -c2

7. More information about program functions and assembler directives

can be found in the MANUAL.DOC file. You can view the MANUAL.DOC

file within the program by pressing F1.

IV. Connecting hardware

�����������������������

You may use IDE11 to write and assemble HC11 source files without

connecting any hardware - but that is no big deal. The real advantage

of IDE11 is the serial connection to a target device.

To connect your PC to a HC11 target, you should use a serial cross

over (null modem) cable looking like this:

Host-PC    Host          Target   Target        To HC11

Sub-D9F   Signal         Signal   Sub-D9F      Port/Pin:

Pin-No.    Name           Name    Pin-No.         (*)

--------------------------------------------------------

3       TxD   <----->   RxD       2            PD0

2       RxD   <----->   TxD       3            PD1

5       GND   <----->   GND       5            VSS

4       DTR   <----->   DSR       6          RESET(#)

7       RTS   <----->   DCD       1           MODE(+)

--------------------------------------------------------

(*): There must be a RS232/TTL transceiver between,

e.g. MAX232!

(#): Optional connection to provide automatic remote reset

by the Host-PC, should be decoupled through a schottky

type diode (or an open drain RS232 transceiver)

(+): Optional connection to provide automatic mode switching

by the Host-PC (Default: Connect MODA+MODB to ground)

Again: Don't forget to tie HC11 pins MODA and MODB to ground (to

establish Special Bootstrap Mode).

V. Registering:

���������������

You know, we need your support to be able to continue improving

IDE11. First, we need your response to learn about your needs

and to detect bugs in the existing release. Another thing is that

(of course!) we do programming for fun, *but*: NOT ONLY for fun ;-).

So, please take a look on the ORDER.DOC file to learn more about

how to register.

VI. Contacting the author:

��������������������������

MCT Lange & Thamm

Mikrocomputertechnik GbR

Hohe Str. 9-13

D-04107 Leipzig

Germany

Tel.: +49-(0)341-2118354

Fax:  +49-(0)341-2118355

Email: mctgbr@aol.com