����������������������������������������������������������������������ͻ

� IDE11 V2.30 - Integrated Development Environment for 68HC11          �

����������������������������������������������������������������������Ķ

� Copyright (C)1994-1996 by MCT Lange & Thamm Mikrocomputertechnik GbR �

� All Rights Reserved                                                  �

����������������������������������������������������������������������ͼ

This file contains a short overview and description of the most

important IDE11 features. Information on software installation

and connection of target hardware can be found in the README.DOC.

Assembler Directives:

���������������������

Directives are operations that do not generate instructions for the

CPU but affect the assembler in certain ways. The following list

contains the most often used assembler directives for IDE11 (always

shown as an example):

����������������������������������������

ORG $B600

����������������������������������������

Sets the program counter to the specified address. Default address

is $0000. Multiple ORGs within one program must appear in ascending

order.

����������������������������������������

name1   EQU $80

name2   EQU name1 + 1

����������������������������������������

Assigns a value to the label "name1". Note: Labels are case sensitive

by default, so "name1" and "NAME1" are different labels!

����������������������������������������

label1  DB  $A0

DB  1,2,3,4,5,6

label2  DB  "Hello message"

label3  DB  "Hello", $20, "message", 0

����������������������������������������

Allocates bytes and initializes them with the given values. Strings

are allowed, even mixing strings and numeric values. Labels are

optional. You may check the generated code by taking a look on the

listing file.

����������������������������������������

label   DW  $E000

DW  main

����������������������������������������

Same as DB but uses word values. Good for specifying interrupt

vectors etc. The label is optional.

����������������������������������������

label   DL  $12345678

����������������������������������������

Same as DB but uses long values (4 byte, most significant byte first).

����������������������������������������

label   RMB 20

����������������������������������������

Reserves the specified number of bytes without assigning a value.

The label is optional.

����������������������������������������

label   RMW 20

����������������������������������������

Same as RMB but uses word values. The example reserves 20*2=40 bytes.

����������������������������������������

label   RML 20

����������������������������������������

Same as RMB but uses long values. The example reserves 20*4=80 bytes.

����������������������������������������

INCLUDE "MYSTUFF.A"

INCLUDE "C:\XYZ\ANOTHER.A"

INCLUDE <HC11AE.H>

����������������������������������������

Includes the given file into the current source file. The line of

the current file will be replaced by the lines of the include file.

The assembler searches the file MYSTUFF.A in the current directory.

The assembler searches the file ANOTHER.A in the specified path.

The third version (use of <...>) tells the assembler to search in

the default include directory (as specified in the Options/Assembler

dialog box). You should set the standard include path option to the

directory which contains your include files (e.g. C:\IDE11\LIB).

����������������������������������������

NOLIST

LIST

����������������������������������������

Disables / enables the generation of the assembly listing for the

following source lines. Default: enabled.

Note: You must allow the creation of an assembly listing file by

checking the appropiate box in the Options/Assembler dialog.

Alternate Assembler Directives:

�������������������������������

For compatibility reasons you may alternatively use the following

assembler directives:

Preferred  Alternatives

�������������������������������������������������������������

ORG       .ORG

EQU       .EQU

RMB       .DS  DEFS  DS  DS.B

RMW       .DS.W

RML       .DS.L

DB        .DB  DEFB  DC.B  BYTE  .BYTE  FCB  STRING  DCC  FCC

DW        .DW  DEFW  DC.W  WORD  FDB

DL        DC.L

INCLUDE   .INCLUDE  INCL  LIB  #INCLUDE (1)

LIST      .LIST

NOLIST    .NOLIST

Note (1): #INCLUDE must start in the first text column!

The following directives will be ignored (no error will be displayed):

BSS  TEXT  DATA  END  .END

Assembler directives are not case sensitive.

Comments:

���������

Comments can start anywhere in the source line. They start with the

";" character or with a double slash "//".

Alternatively the "*" character is allowed provided the comment starts

in the first (very left) text column. At all other positions "*" is

considered to be a multiply operator.

Another way is to embrace a text passage in c-style comment, e.g.:

label  /* remark */  JSR  sub1

Labels:

�������

Labels are case sensitive and have up to 31 significant characters.

Allowed character: "a"-"z", "A"-"Z", "0"-"9", "@", "_" and ".". The

first character must not be a number or ".". Labels can optionally

end with a colon ":" (ignored by assembler).

Instructions:

�������������

Refer to the HC11 Reference Manual for details on instructions.

The INSTRUCT.DOC file contains a table with all HC11 instructions.

Addressing Modes:

�����������������

The notation of operands depending of the addressing mode is given

here by examples:

Addr. Mode   Examples

���������������������������������������

Immediate    #$20  #123  #'A'  #isr_vec

Direct       <$0000  <VAR1

Extended     $A000  2400  main

Indexed      0,X  2,y

Relative     label1

Some confusion exists concerning the BSET/BCLR and BRSET/BRCLR

instructions. Here are some examples how to use them:

BSET  0,x %00010000

BCLR  0,y FLAG_BIT

BSET  $2A $80

here    BRSET 0,x 1 here

wait    BRCLR 0,y $80 wait

BRSET flag $FF main

This is identical to the description in the HC11 Reference Manual.

Constants:

����������

Numerical constants may be decimal (e.g. 1, 99, 800000), hexadecimal

(e.g. $2C, $8000, $9abcde) or binary (e.g. %00101100).

Character constants have the form 'A'. Example:

LDAA #'z'

String constants look like this: "String constant". This can be used

in conjunction with a DB directive:

DB  "Hello, world!"

Operators:

����������

Valid operators are: "+", "-", "*" and "/". Complex expressions may

be freely build using "(" and ")".

Information of Target Settings (Options/Target/Detail):

�������������������������������������������������������

--> MCU type

There are many different HC11 family members, it is necessary to tell

IDE11 which one is used. The only difference between 'A8, 'A1 and 'A0

is the presence of EEPROM and/or ROM: The 'A8 has EEPROM *and* ROM,

the 'A1 has EEPROM, but no ROM and the 'A0 has none of them. The same

scheme is used with 'E9, 'E1 and 'E0. The HC812E2 differs from 'A1

concerning size and location of EEPROM. It is the only member with 2KB

on chip EEPROM. The F1 is an advanced family member with an

unmultiplexed external address/data bus.

--> External memory

IDE11 can handle RAM and/or EEPROM as external memory. If your target

operates in Expanded Multiplexed Mode (external memory connected) you

must set the correct memory type, split into a lower and an upper

memory area. Remark: You must ALWAYS set your target to Special

Bootstrap Mode initially. If it is necessary to change to Expanded

Mode, IDE11 will do it for you, depending on the setting:

--> Mode ctrl

Default: None

Normally no (hardware) Mode Control is necessary, because IDE11 can

use a software command to switch from Special Bootstrap Mode to

Expanded Mode. But sometimes it is more realistic to switch the

MODA+MODB pins. This can be done manually or using the output /RTS

of the Host PC. Pay attention to the TTL/RS232 converter which

is neccessary when using /RTS line.

--> Reset ctrl

Default: /DTR=L

It is highly recommended to let IDE11 reset your target automatically

when necessary. This (for example) happens when a download starts,

when you access the CONFIG Register and at the moment a program is

executed. You also can invert the reset line. If you choose the option

/DTR=H the target will receive an active high reset pulse. See also

comments on cabling in the README.DOC file.

If you choose the option Manual you will have to reset your target

manually each time IDE11 asks you to do so.